Vtoplevel.h

- Verilator-generated “SystemC wrapper”.

- Presents the complete set of model ports and binds them to the internal design; primary entry point for a SystemC testbench.

- Declares ports plus the usual helpers: constructor/destructor, eval_step(), final(), trace(), etc.

- Bridges external interfaces to internal state, schedules evaluation, and provides waveform support.

Vtoplevel__Syms.h

- Acts as the model’s “entry router”, connecting the outer SystemC wrapper to the internal RTL state.

- Maintains the model pointer, root-instance handle, simulation-context settings, and other globals (waveform flags, time control, deferred delete queue). Without it, TOP cannot be constructed correctly nor share runtime resources.

Vtoplevel___024root.h

- Concrete hardware implementation class; holds all ports, registers, trigger bitmaps, and per-module internals derived from the RTL.

Vtoplevel__pch.h

- Precompiled header bundling common includes to speed up compilation.

- Verilator runtime headers

- verilated_sc.h, verilated.h (and companions such as verilated_vcd_sc.h) provide the core simulation API, tracing, and utility macros used by the generated model.

Vtoplevel___024root__DepSet_h9890b585__0sc.cpp function

- Vtoplevel___024root___eval_ico – Check the __VicoTriggered bitmap, fire _ico_sequent__TOP__0, and flag trace activity.

- Vtoplevel___024root___ico_sequent__TOP__0 – Sample SystemC ports into __Vcellinp*, drive sdram_clk_pad_o, update double-click reset (RTL input comb. logic).

- Vtoplevel___024root___eval_phase__ico – Trigger-dispatch entry; recompute ICO triggers and run eval_ico when needed.

- Vtoplevel___024root___eval_act – Placeholder for the active region (no body in this split; triggers handled elsewhere).

- Vtoplevel___024root___eval_nba – Iterate __VnbaTriggered bits, calling _nba_sequent__TOP__0…4 and updating __Vm_traceActivity.

- _nba_sequent__TOP__0 – System-clock domain: 1 MHz divider, FIFO CDC sync, SDRAM state/command registers, pad driving.

- _nba_sequent__TOP__1 – 1 MHz domain: double-click FSM, LED counter, FIFO write-side packing, GPIO display.

- _nba_sequent__TOP__2 – 1 MHz domain: button edge tracking (btn_last/now/down).

- _nba_sequent__TOP__3 – System-clock domain: apply __Vdly__ updates, handle SDRAM/FIFO handshakes, capture read data, generate next state/command, bank/address outputs, DQ drive.

- _nba_sequent__TOP__4 – Commit CDC sync registers (rd_syn2, wr_syn2, …) calculated in the 1 MHz domain.

- Vtoplevel___024root___eval_phase__act – Active-phase scheduler; invokes _eval_triggers__act, merges with NBA triggers, calls eval_act.

- Vtoplevel___024root___eval_phase__nba – NBA-phase scheduler; runs eval_nba when any bit is set, then clears the bitmap.

- Vtoplevel___024root___eval – Main evaluation loop executing ICO → ACT/NBA iterations with 0x64 safeguard iterations; backing implementation of eval_step().

Other DepSet / Slow

Vtoplevel___024root__DepSet_h9890b585__0__Slowsc.cpp

- Implements cold-path routines: eval_static, eval_initial, eval_settle, _stl_sequent__TOP__0 for initial combinational settling, plus trace-activity reset helpers.

Vtoplevel___024root__DepSet_h887c3000__0.cpp

- Provides trigger evaluators for ICO/ACT regions (_eval_triggers__ico, _eval_triggers__act), tracking clock edges and first-iteration flags.

Vtoplevel___024root__DepSet_h887c3000__0__Slow.cpp

- Slow-path settle trigger evaluator (_eval_triggers__stl) used during initial convergence.

Vtoplevel___024root__Slow.cpp

- Houses high-level cold functions (eval_static, eval_initial, eval_final, trace registration entry points) shared by multiple DepSets.